LAB NUMBER 2

ELASTIC PROPERTIES OF RUBBER

INSTRUCTIONS FOR EXPERIMENTS

ANAND M. SHARAN

SEPT 18, 2009

EXPERIMENTAL DESCRIPTION

1.	Fig. 1  shows  weights suspended from O Ring.

2.	The suspended weights cause stretching or the lengthening of the O-Ring . The Weight or the 	vertical force , F =  m g ,   is balanced by tension = 2T  in the both parts of the O-Ring

3.        There are three weights ( masses )  of magnitudes shown in Fig. 3.

4  	Start without any of the masses  suspended and  attach Two clips to the O-Ring as shown in 	Fig. 1.

5. 	Measure the length between the clips using a Vernier Caliper ( mm ) , and the diameter of the O-	Ring 	( mm ) using a micrometer

6 	Add masses one at a time and correspondingly -  measure the length between the clips and 	diameter of the O-Ring and enter the data  in the table below.

| Number | Masses Added ( g ) | Total Mass ( g ) | Distance Between Clips ( mm ) | Diameter of O-Ring ( mm ) |
| --- | --- | --- | --- | --- |
| 1 | 0 | 53 (Mass of the Assembly ) |  |  |
| 2 | 1007 | 1060 |  |  |
| 3 | 1018 |  |  |  |
| 4 | 1021 |  |  |  |
| 5 | -1021 |  |  |  |
| 6 | -1018 |  |  |  |
| 7 | -1007 |  |  |  |

Resolution of the Mass = 1 gram.

- Enter Your  data ( ONLY IN THE SHADED AREA ) in the Computer Software. Your data should be  different from those already in the computer .   There are two worksheets in this software. The first one is  ‘ data ‘ and the second one is called   ‘ calculation ‘.

8 Pl be careful to enter your the uncertainty values in the second ( calculation ) worksheet

Data Worksheet

Calculation Worksheet

| Uncertainty of the Measurement tool for Diameter | 0.002 | (mm) |  |
| --- | --- | --- | --- |
| Uncertainty of the Measurement tool for Length | 1 | (mm) |  |
| Uncertainty of the Measurement tool for Length | 0.0098 | (N) |  |

- Carry out one set of sample calculations

- Report on  ( a ) Modulus of Elasticity, E, ( b )  Poisson’s Ratio ν,  using the plotted graphs and ( c )  Uncertainties  associated with each of the two ( E, and ν )

- Report Conclusions