7.41 An uncold-worked brass specimen of average grain size 0.008 mm has a yield strength of 160 MPa (23,500 psi).  Estimate the yield strength of this alloy after it has been heated to 700C for 1000 s, if it is known that the value of ky is 12.0 MPa-mm1/2 (1740 psi-mm1/2).

Solution

In order to solve this problem, it is first necessary to calculate the constant σ0 in Equation 7.7 ( Page 213 ) as

s0=sy−kyd-1/2

=160MPa−(12.0MPa−mm1/2)(0.008mm)−12=25.8MPa(4046 psi)

Next, we must determine the average grain size after the heat treatment.  From Figure 7.25 ( Page 225 ) at 7000 C after 1000 s (16.7 min) the average grain size of a brass material is about 0.20 mm.  Therefore, calculating σy at this new grain size using Equation 7.7 we get

sy=s0+kyd-1/2

=25.8 MPa+(12.0MPa-mm1/2)(0.20mm)-1/2=52.6MPa(7940 psi)