CLASS_SOLUTION_CH8

8.14  A fatigue test was conducted in which the mean stress was 50 MPa (7250 psi) and the stress amplitude was 225 MPa (32,625 psi).

(a) Compute the maximum and minimum stress levels.

(b) Compute the stress ratio.

(c) Compute the magnitude of the stress range.

Solution

(a)  Given the values of m (50 MPa) and a (225 MPa) we are asked to compute max and min.  From Equation 8.14

sm=smax+smin2=50 MPa

Or,

max + min = 100 MPa

Furthermore, utilization of Equation 8.16 yields

sa=smax−smin2=225 MPa

Or,

max – min = 450 MPa

Simultaneously solving these two expressions leads to

smax=275 MPa (40,000 psi)

smin=−175 MPa (−25,500 psi)

(b)  Using Equation 8.17 the stress ratio R is determined as follows:

R=sminsmax=−175MPa275MPa=−0.64

(c)  The magnitude of the stress range r is determined using Equation 8.15 as

sr=smax−smin=275 MPa −(−175 MPa)=450 MPa  (65,500 psi)