Tables & Graphs

Given a set of observations { x1, x2, ..., xn }, one can summarize using:

Guidelines:

	time series plot

	stem and leaf

	frequency table

	bar chart

	histogram

	pie chart

	pictogram (or other methods)

Example 2.01 	Course text (Devore, seventh edition), ex. 1.2 p. 23 q. 24, modified

[This data set is available from the course web site, at

www.engr.mun.ca/~ggeorge/3423/Minitab/s01DescStat/index.html]

The data set below consists of observations on shear strengths  x (in pounds) of ultrasonic spot welds made on a certain type of alclad sheet.

5434	4948	4521	4570	4990	5702	5241	5112	5015	4659	4806	4637

5670	4381	4820	5043	4886	4599	5288	5299	4848	5378	5260	5055

5828	5218	4859	4780	5027	5008	4609	4772	5133	5095	4618	4848

5089	5518	5333	5164	5342	5069	4755	4925	5001	4803	4951	5679

5256	5207	5621	4918	5138	4786	4500	5461	5049	4974	4592	4173

5296	4965	5170	4740	5173	4568	5653	5078	4900	4968	5248	5245

4723	5275	5419	5205	4452	5227	5555	5388	5498	4681	5076	4774

4931	4493	5309	5582	4308	4823	4417	5364	5640	5069	5188	5764

5273	5042	5189	4986

(a)	Produce a stem and leaf display of the data.

(b)	Construct a bar chart of the data, using ten class intervals of equal width, with the first interval having lower limit 4000 (inclusive) and upper limit 4200 (exclusive). [Such a bar chart will be consistent with one that appeared in the paper “Comparison of Properties of Joints Prepared by Ultrasonic Welding and Other Means”, J. Aircraft, 1983, pp. 552-556.]

By itself, this table is not very helpful as we try to grasp the overall picture of shear strengths.

One way to improve visibility is simply to rearrange these data into ascending order:

4173	4308	4381	4417	4452	4493	4500	4521	4568	4570	4592	4599

4609	4618	4637	4659	4681	4723	4740	4755	4772	4774	4780	4786

4803	4806	4820	4823	4848	4848	4859	4886	4900	4918	4925	4931

4948	4951	4965	4968	4974	4986	4990	5001	5008	5015	5027	5042

5043	5049	5055	5069	5069	5076	5078	5089	5095	5112	5133	5138

5164	5170	5173	5188	5189	5205	5207	5218	5227	5241	5245	5248

5256	5260	5273	5275	5288	5296	5299	5309	5333	5342	5364	5378

5388	5419	5434	5461	5498	5518	5555	5582	5621	5640	5653	5670

5679	5702	5764	5828

An additional improvement to the visual appearance is the stem and leaf display.   Part of the output from a MINITAB session, using default values, is reproduced on the left hand side below.   The left-most column is a cumulative frequency count from the nearer end. Note how MINITAB returns only the thousands and hundreds digits in the stem and the tens digit in the leaf.   The units digit is truncated (lost altogether).   On the right hand side (to the right of the comment markers ###) is shown a manual version that retains both digits of the leaf.

Stem-and-leaf of Shear st  N  = 100

Leaf Unit = 10

### Manual version, retaining

### both digits of the leaf:

###   Stem Leaf

1   41 7			###    41   73

1   42				###    42

3   43 08			###    43   08 81

6   44 159			###    44   17 52 93

12   45 026799		###    45   00 21 68 70 92 99

17   46 01358 		###    46   09 18 37 59 81

24   47 2457788		###    47   23 40 55 72 74 80 86

32   48 00224458		###    48   03 06 20 23 48 48 59 86

43   49 01234566789	###    49   00 18 25 31 48 51 65 68 74 86 90

(14)  50 00124445667789	###    50   01 08 15 27 42 43 49 55 69 69 76 78 89 95

43   51 13367788		###    51   12 33 38 64 70 73 88 89

35   52 00124445677899	###    52   05 07 18 27 41 45 48 56 60 73 75 88 96 99

21   53 034678		###    53   09 33 42 64 78 88

15   54 1369			###    54   19 34 61 98

11   55 158			###    55   18 55 82

8   56 24577			###    56   21 40 53 70 79

3   57 06			###    57   02 64

1   58 2			###    58   28

With the "Increment" option of "Graph > Stem-and-leaf" set to 200 instead of the default value of 100, the number of stems is reduced to the appropriate number, namely .   MINITAB’s output is then

Stem-and-leaf of Shear st  N  = 100

Leaf Unit = 100

1    4 1

3    4 33

12    4 444555555

24    4 666667777777

43    4 8888888899999999999

(22)   5 0000000000000011111111

35    5 22222222222222333333

15    5 4444555

8    5 6666677

1    5 8

Notice how the leaf unit is now 100, not 10, so that the last two digits of each value are now lost.

From this stem and leaf display it is easy to generate a frequency table for shear stress manually, in the form required by part (b) of the question.

Interval 		     Frequency

for  x			 f

4000  x < 4200		  1

4200  x < 4400		  2

4400  x < 4600		  9

4600  x < 4800		12

4800  x < 5000		19

5000  x < 5200		22

5200  x < 5400		20

5400  x < 5600		  7

5600  x < 5800		  7

5800  x < 6000		  1

_____

Total:	          100

The bar chart generated by MINITAB (which it calls an "histogram") also provides the frequency table:

There is a subtle difference between a “bar chart” and an “histogram”.   A bar chart is used for discrete (countable) data (such as “number of defective items found in one run of a process”) or nonnumeric data (such as “engineering discipline chosen by students”).   The bars are drawn with arbitrary (often equal) width.   No two bars should touch each other.   The height of each bar is proportional to the frequency.

An histogram is used for continuous data (such as “shear stress” or “weight” or “time”, where between any two possible values another possible value can always be found).    [An histogram can also be used for discrete data.]    Each bar covers a continuous interval of values and just touches its neighbouring bars without overlapping.    Every possible value lies in exactly one interval.    Unlike a bar chart, it is the area of each bar that is proportional to the frequency in that interval.    Only if all intervals are of equal width will the histogram have the same shape as the bar chart.

The relative frequency in an interval is the proportion of the total number of observations that fall inside that interval.    A relative frequency histogram can then be generated, with bar height given by

Relative Frequency             Frequency

Bar height  =  ------------------  =  --------------------------.

Class Width         (Total Freq.)*(Class Width)

The total area of all bars in a relative frequency histogram is always 1.    In chapter 8 we will see that the relative frequency histogram is related to the graph of a probability density function, the total area under which is also 1.

For the example above, the total frequency is 100 and the class width is 200, so the height of each bar in the relative frequency histogram is given by

Frequency      Frequency

Bar height  =  ---------   =  --------- .

100 * 200       20 000

The cumulative frequency is the sum of all frequencies up to and including the current class.

Extending the previous table:

Relative 	Height of	    Cumulative

Interval	    Frequency	     Frequency 	histogram	     Frequency

for   x		f		  r	    bar			 c

4000  x < 4200	  1		.01	.00005			  1

4200  x < 4400	  2		.02	.00010			  3

4400  x < 4600	  9		.09	.00045			12

4600  x < 4800	12		.12	.00060			24

4800  x < 5000	19		.19	.00095			43

5000  x < 5200	22		.22	.00110			65

5200  x < 5400	20		.20	.00100			85

5400  x < 5600	  7		.07	.00035			92

5600  x < 5800	  7		.07	.00035			99

5800  x < 6000	  1		.01	.00005		          100

_____	         _____

Total:       100	          1.00

The relative frequency histogram is on the next page.

Relative frequency histogram for the set of 100 observations of shear strengths (in pounds) of ultrasonic spot welds made on a certain type of alclad sheet.

From this diagram, the relative frequency of any class can be recovered by calculating the area of the bar.    For example, the relative frequency of the class  4800  x < 5000  is given by

relative frequency  =  area of bar  =

If you are absent from the first Minitab tutorial, then view the web page

www.engr.mun.ca/~ggeorge/3423/Minitab/s01DescStat/index.html

carefully.

Measures of Location

The mode is the most common value.

In example 2.01 the mode is   			________________________

From the frequency table, the modal class  is 	________________________

A disadvantage of the mode as a measure of location is

________________________________________________________________________

________________________________________________________________________

The sample median   x  (or the population median  μ) is the “halfway value” in an ordered set.   For   n   data, the median is the (n + 1)/2 th value if  n  is odd.

The median is the semi-sum of the two central values if  n  is even,

(that is median = [ (n/2 th value) + ((n/2 + 1)th value) / 2 ).

For the example above,

sample median   x =

In the table of grouped values, the 50th and 51st values fall in the same class.

The median class is therefore

The sample arithmetic mean  x (or the population mean   ) is the ratio of the sum of the observations to the number of observations.

From individual observations, 	x =

and from a frequency table, 		x =

For the example above, from the 100 raw data (not from the frequency table),

x =

The relative advantages of the mean and the median can be seen from a pair of smaller samples.

Example 2.02

Let   A  =  { 1, 2, 3, 4, 5 }	and	B  =  { 1, 2, 3, 23654, 5 } .

Then

x   = 			for set A     and   x   = 		for set B , while

x = 		for set A     and    x = 		for set B.

Note that the mode is not well defined for either set.

A disadvantage of the mean as a measure of location is

Advantages of the mean over the median include

- the median uses only the central value(s) while the mean uses all values.

For a symmetric population, the mean     and the median μ  will be equal.    If the mode is unique, then it will also be equal to the mean and median of a symmetric population.

Measures of Variation

The simplest measure of variation is the  range  =  (largest value  smallest value).

A disadvantage of the sample range is

A disadvantage of the population range is

The effect of outliers can be eliminated by using the distance between the quartiles of the data as a measure of spread instead of the full range.

The lower quartile  qL  is the  { (n + 1) / 4 }th smallest value.

The upper quartile  qU  is the  { 3(n + 1) / 4 }th smallest value.

[Close relatives of the quartiles are the fourths.

The lower fourth is the median of the lower half of the data, (including the median if and only if the number  n  of data is odd).

The upper fourth is the median of the upper half of the data, (including the median if and only if the number  n  of data is odd).

In practice there is often little or no difference between the value of a quartile and the value of the corresponding fourth.]

The interquartile range is   IQR  = qU   qL    and

the semi-interquartile range is   SIQR  = (qU   qL) / 2

Example 2.01:

n  =  100        (n + 1) / 4  =  25.25        qL  =  value 1/4 of the way from x25 to x26

=

and  3 (n + 1) / 4  =  75.75        qU  =  value 3/4 of the way from x75 to x76

=

The semi-interquartile range is then

The boxplot illustrates the median, quartiles, outliers and skewness in a compact visual form.

The boxplot for example 2.01, as generated by an older version of MINITAB, is shown below.

[See the tutorial session for a more modern version of this output.]

MTB > BoxPlot C1.

---------------

------------------I      +      I----------------

---------------

------+---------+---------+---------+---------+---------+C1

4200      4550      4900      5250      5600      5950

Unequal whisker lengths reveal skewness.   The whiskers extend as far as the last observation before the inner fence.   The fences are not plotted by MINITAB.

The inner fences are 1.5 interquartile ranges beyond the nearer quartile, at

xL   1.5 IQR  (lower)       and	xU  + 1.5 IQR  (upper)   [4097.625 and 5980.625 here]

The outer fences are twice as far away from the nearer quartile, at

xL    3 IQR  (lower)         and	xU  + 3 IQR     (upper)	  [3391.500  and  6686.750 here]

Any observations between inner & outer fences are mild outliers, which would be indicated by an open circle (or, in MINITAB, by an asterisk).  There are no outliers in this example.

Any observations beyond outer fences are extreme outliers, which would be indicated by a closed circle (or, in MINITAB, by an asterisk or a zero).

If you encounter an extreme outlier, then check if the measurement is incorrect or is from a different population.    If the observation is genuine, then it is a rare event (< 0.01% in most populations).

Measures of variability based on quartiles are not easy to manipulate using calculus methods.

The deviation of the  ith observation from the sample mean is xi − x .   At first sight, one might consider that the sum of all these deviations could serve as a measure of variability.

However:

An alternative is the mean absolute deviation from the mean, defined as

MAD = 1n i=1n  xi − x

Unfortunately, the function f(x) =  xi − x  is not differentiable at the one point where the derivative is most needed, at  x = x.   Instead, the mean square deviation from the mean is used:

The population variance  2  for a finite population of  N  values is given by

σ2 = 1N i=1N xi − μ2

and the sample variance  s2  of a sample of  n  values is given by

s2 = 1n−1 i=1n xi − x2

The square root of a variance is called the standard deviation and is positive (unless all values are exactly the same, in which case the standard deviation is zero).     The reason for the different divisor (n   1)  in the expression for the sample variance  s2  will be explained later.

The MINITAB output for various summary statistics for example 2.01 is shown here:

MTB > Describe C1

N     MEAN   MEDIAN   TRMEAN   STDEV   SEMEAN

C1      100   5049.2   5052.0   5050.5   351.5     35.1

MIN      MAX       Q1       Q3

C1   4173.0   5828.0   4803.8   5274.5

When calculating a sample variance by hand or on some hand held calculators, one of the following shortcut formulæ may be easier to use:

s2 =  i=1nxi 2   −  1ni=1nxi2n−1	or	s2 = i=1nxi2 − n x2n−1	or

s2 =  ni=1nxi 2 − i=1nxi2n (n−1) .

For integer values of  x,  the last of these three formulæ allows the sample variance to be expressed exactly as a fraction.   The formulæ for data taken from a frequency table with  m  classes are similar:

s2 = 1n−1 i=1m fi xi − x2	or	s2 = i=1m fi xi 2 − 1n i=1m fi xi2n−1

or	s2 = i=1m fi xi 2 − n x2n−1		or	s2 = n i=1m fi xi 2 − i=1m fi xi2n n−1

where, in each case,   n = i=1m fi		and	x = i=1m fi xii=1m fi .

However, all of the shortcut formulæ are more sensitive to round-off errors than the definition is.

Example 2.03:

Find the sample variance for the set  { 100.01, 100.02, 100.03 } by the definition and by one of the shortcut formulæ, in each case rounding every number that you encounter during your computations to six or seven significant figures, (so that 100.012 = 10002.00 to 7 s.f.).    The correct value for  s2 in this case is .0001, but rounding errors will cause all three shortcut formulæ to return an incorrect value of zero.   (Try it!).

Example 2.04:

Find the sample mean and the sample standard deviation for

x = the number of service calls during a warranty period, from the frequency table below.

xi	 	   fi		fi xi		fi  xi2

0		  65

1		  30

2		    3

3		    2

Sum:	100

x =  fi xifi =

s2 = n  fi xi 2 −  fi xi2n n−1   =

or

s2 = 1n−1  fixi − x2 =

For any data set:

  3/4 of all data lie within two standard deviations of the mean.

  8/9 of all data lie within three standard deviations of the mean.

  (1  1/k2 ) of all data lie within  k standard deviations of the mean (Chebyshev’s inequality).

For a bell-shaped distribution (for which population mean = population median = population mode):

~ 68% of all data lie within one standard deviation of the mean.

~ 95% of all data lie within two standard deviations of the mean.

> 99% of all data lie within three standard deviations of the mean.

Misleading Statistics  -   Example 2.05

Both graphs below are based on the same information, yet they seem to lead to different conclusions.

“Our profits rose enormously in the 		     vs. 		“Our profits rose by only 10%

last quarter.”							  in the last quarter.”

Visual displays can be very misleading.    Questions to ask when viewing visual summaries of data include,

for graphs:

for bar charts / pictograms :

[End of the chapter “Descriptive Statistics”]

[Space for any additional notes]