4.	Partial Differential Equations

Partial differential equations (PDEs) are equations involving functions of more than one variable and their partial derivatives with respect to those variables.

Most (but not all) physical models in engineering that result in partial differential equations are of at most second order and are often linear.   (Some problems such as elastic stresses and bending moments of a beam can be of fourth order).   In this course we shall have time to look at only a very small subset of second order linear partial differential equations.

Sections in this Chapter:

4.1	Major Classifications of Common PDEs

4.2	The Wave Equation – d’Alembert Solution

4.3	The Wave Equation – Vibrating Finite String

4.4	The Maximum-Minimum Principle

4.5	The Heat Equation

4.1	Major Classifications of Common PDEs

A general second order linear partial differential equation in two Cartesian variables can be written as

Three main types arise, based on the value of  D  =  B 2 – 4AC (a discriminant):

Hyperbolic, wherever (x, y) is such that  D > 0;

Parabolic, wherever (x, y) is such that  D = 0;

Elliptic, wherever (x, y) is such that  D < 0.

Among the most important partial differential equations in engineering are:

The wave equation:

or its one-dimensional special case  [which is hyperbolic everywhere]

(where u is the displacement and c is the speed of the wave);

The heat (or diffusion) equation:

a one-dimensional special case of which is

[which is parabolic everywhere]

(where u is the temperature, μ is the specific heat of the medium, ρ is the density and K is the thermal conductivity);

The potential (or Laplace’s) equation:

a special case of which is    [which is elliptic everywhere]

The complete solution of a PDE requires additional information, in the form of initial conditions (values of the dependent variable and its first partial derivatives at t = 0), boundary conditions (values of the dependent variable on the boundary of the domain) or some combination of these conditions.

4.2	d’Alembert Solution

Example 4.2.1

Show that

is a solution to the wave equation

with initial conditions   y(x, 0)  =  f (x)  and

for any twice differentiable function  f (x).

Example 4.2.1    (continued)

A more general d’Alembert solution to the wave equation for an infinitely long string is

This satisfies the wave equation

and

Initial configuration of string:		y(x, 0)  =  f (x)   for

and

Initial speed of string:

for any twice differentiable functions  f (x)  and  g(x).

Physically, this represents two identical waves, moving with speed c in opposite directions along the string.

Proof that  satisfies both initial conditions:

Using a Leibnitz differentiation of the integral:

Example 4.2.2

An elastic string of infinite length is displaced into the form  y = cos  x/2  on [–1, 1] only (and y = 0 elsewhere) and is released from rest.   Find the displacement  y(x, t) at all locations on the string  and at all subsequent times (t > 0).

See the web page "www.engr.mun.ca/~ggeorge/5432/demos/ex422.html" for an animation of this solution.

Example 4.2.2    (continued)

Some snapshots of the solution are shown here:

A more general case of a d’Alembert solution arises for the homogeneous PDE with constant coefficients

The characteristic (or auxiliary) equation for this PDE is

This leads to the complementary function (which is also the general solution for this homogeneous PDE)

where

and	D  =  B 2 – 4AC

and  f1, f2  are arbitrary twice-differentiable functions of their arguments.

λ 1  and  λ 2  are the roots (or eigenvalues) of the characteristic equation.

In the event of equal roots, the solution changes to

where  h(x, y)  is any non-trivial linear function of x and/or y (except  y + λx).

The wave equation is a special case with  y = t, A = 1, B = 0, C = –1/c2 and λ = ± 1/c.

Example 4.2.3

u(x, 0)  =  x2

uy(x, 0)  =  0

(a)	Classify the partial differential equation.

(b)	Find the value of  u  at  (x, y)  =  (0, 1).

(a)	Compare this PDE to the standard form

(b)

Example 4.2.3    (continued)

Example 4.2.4

Find the complete solution to

u(x, 0)  =  2x + 1 ,

uy(x, 0)  =  4  6x .

This PDE is non-homogeneous.

For the particular solution, we require a function such that the combination of second partial derivatives resolves to the constant 14.   It is reasonable to try a quadratic function of x and y as our particular solution.

Try	u  =  ax2 + bxy + cy2

Example 4.2.4    (continued)

Example 4.2.5

Find the complete solution to

u = 0   on   x = 0 ,

u =  x2   on   y = 1 .

Two-dimensional Laplace Equation

A function f (x, y) is harmonic if and only if  2f = 0  everywhere inside a domain .

### Example 4.2.6

Is  u  =  ex sin y  harmonic on 2?

4.3	The Wave Equation – Vibrating Finite String

The wave equation is

If u(x, t) is the vertical displacement of a point at location x on a vibrating string at time t, then the governing PDE is

If u(x, y, t) is the vertical displacement of a point at location (x, y) on a vibrating membrane at time t, then the governing PDE is

or, in plane polar coordinates (r, θ), (appropriate for a circular drum),

Example 4.3.1

An elastic string of length L is fixed at both ends (x = 0 and x = L).   The string is displaced into the form  y = f (x)  and is released from rest.   Find the displacement  y(x, t) at all locations on the string (0 < x < L) and at all subsequent times (t > 0).

The boundary value problem for the displacement function  y(x, t)  is:

Both ends fixed for all time:

Initial configuration of string:

String released from rest:

Example 4.3.1   (continued)

Separation of Variables (or Fourier Method)

Attempt a solution of the form   y(x, t)  =  X(x) T(t)

Substitute  y(x, t)  =  X(x) T(t)  into the PDE:

Example 4.3.1   (continued)

Therefore our complete solution is

Example 4.3.1   (continued)

This solution is valid for any initial displacement function  f (x)  that is continuous with a piece-wise continuous derivative on [0, L] with  f (0) = f (L) = 0.

If the initial displacement is itself sinusoidal , then the complete solution is a single term from the infinite series,

.

Suppose that the initial configuration is triangular:

Then the Fourier sine coefficients are

Example 4.3.1   (continued)

See the web page "www.engr.mun.ca/~ggeorge/5432/demos/ex431.html" for an animation of this solution.

Example 4.3.1   (continued)

Some snapshots of the solution are shown here:

These graphs were generated from the Fourier series, truncated after the fifth non-zero term.

Example 4.3.2

An elastic string of length L is fixed at both ends (x = 0 and x = L).   The string is initially in its equilibrium state [y(x, 0) = 0 for all x] and is released with the initial velocity .   Find the displacement  y(x, t) at all locations on the string (0 < x < L) and at all subsequent times (t > 0).

The boundary value problem for the displacement function  y(x, t)  is:

Both ends fixed for all time:		y(0, t)  =  y(L, t)  =  0   for   t > 0

Initial configuration of string:		y(x, 0)  =  0   for   0 < x < L

String released with initial velocity:

As before, attempt a solution by the method of the separation of variables.

Substitute  y(x, t)  =  X(x) T(t)  into the PDE:

Again, each side must be a negative constant.

We now have the pair of ODEs

The general solutions are

respectively, where  A, B, C and D are arbitrary constants.

Example 4.3.2   (continued)

Consider the boundary conditions:

For a non-trivial solution, this requires  .

We now have a solution only for a discrete set of eigenvalues , with corresponding eigenfunctions

and

So far, the solution has been identical to Example 4.3.1.

Consider the initial condition   y(x, 0)  =  0 :

The initial value problem for  T(t)  is now

the solution to which is

Our eigenfunctions for y are now

Example 4.3.2   (continued)

Differentiate term by term and impose the initial velocity condition:

which is just the Fourier sine series expansion for the function  g(x).

The coefficients of the expansion are

which leads to the complete solution

This solution is valid for any initial velocity function  g(x)  that is continuous with a piece-wise continuous derivative on [0, L] with  g(0) = g(L) = 0.

The solutions for Examples 4.3.1 and 4.3.2 may be superposed.

Let  y1(x, t)  be the solution for initial displacement  f (x) and zero initial velocity.

Let  y2(x, t)  be the solution for zero initial displacement and initial velocity  g(x).

Then  y(x, t)  =  y1(x, t)  +  y2(x, t)  satisfies the wave equation

(the sum of any two solutions of a linear homogeneous PDE is also a solution),

and satisfies the boundary conditions  y(0, t)  =  y(L, t)  =  0 .

y(x, 0)  =  y1(x, 0)  +  y2(x, 0)  =  f (x)  + 0,

which satisfies the condition for initial displacement  f (x).

yt (x, 0)  =  y1t(x, 0)  +  y2t (x, 0)  =  0 + g(x),

which satisfies the condition for initial velocity  g(x).

Therefore the sum of the two solutions is the complete solution for initial displacement f (x)  and  initial velocity  g(x):

4.4	The Maximum-Minimum Principle

Let    be some finite domain on which a function  u(x, y)  and its second derivatives are defined.   Let   be the union of the domain with its boundary.

Let  m  and M  be the minimum and maximum values respectively of u on the boundary of the domain.

If  2u  0  in  ,  then  u is subharmonic and

If  2u  0  in  ,  then  u is superharmonic and

If  2u = 0  in  ,  then  u is harmonic (both subharmonic and superharmonic) and

u is either constant on  or   m < u < M  everywhere on .

Example 4.4.1

2u = 0  in   : x2 + y2 < 1   and   u(x, y) = 1  on C :  x2 + y2 =  1.

Find  u(x, y)  on .

Example 4.4.2

2u = 0  in the square domain  : –2 < x < +2,  –2 < y < +2 .

On the boundary C, on the left and right edges (x = ±2),  u(x, y) = 4 – y2,

while on the top and bottom edges (y = ±2),  u(x, y) =  x2 – 4.

Find bounds on the value of  u(x, y)  inside the domain .

4.5	The Heat Equation

For a material of constant density , constant specific heat μ and constant thermal conductivity K, the partial differential equation governing the temperature u at any location (x, y, z) and any time t is

Example 4.5.1

Heat is conducted along a thin homogeneous bar extending from x = 0 to x = L.   There is no heat loss from the sides of the bar.  The two ends of the bar are maintained at temperatures T1 (at x = 0) and T2 (at x = L).   The initial temperature throughout the bar at the cross-section x is f (x).

Find the temperature at any point in the bar at any subsequent time.

The partial differential equation governing the temperature u(x, t) in the bar is

together with the boundary conditions

u(0, t)  =  T1    and    u(L, t)  =  T2

and the initial condition

u(x, 0)  =  f (x)

[Note that if an end of the bar is insulated, instead of being maintained at a constant temperature, then the boundary condition changes to  or .]

Attempt a solution by the method of separation of variables.

u(x, t)  =  X(x) T(t)

Again, when a function of t only equals a function of x only, both functions must equal the same absolute constant.   Unfortunately, the two boundary conditions cannot both be satisfied unless T1 = T2 = 0.   Therefore we need to treat this more general case as a perturbation of the simpler (T1 = T2 = 0) case.

Example 4.5.1   (continued)

Let  u(x, t)  =  v(x, t)  +  g(x)

Substitute this into the PDE:

This is the standard heat PDE for v if we choose  g  such that  g"(x) = 0.

g(x)  must therefore be a linear function of x.

We want the perturbation function  g(x)  to be such that

u(0, t)  =  T1  ,    u(L, t)  =  T2

and

v(0, t)  =  v(L, t)  =  0

Therefore  g(x)  must be the linear function for which  g(0)  =  T1  and  g(L)  =  T2 .

It follows that

and we now have the simpler problem

together with the boundary conditions

v(0, t)  =   v(L, t)  =  0

and the initial condition

v(x, 0)  =  f (x)  –  g(x)

Now try separation of variables on  v(x, t) :

v(x, t)  =  X(x) T(t)

But   v(0, t)  =   v(L, t)  =  0       X(0)  =  X(L)  =  0

This requires c to be a negative constant, say .

The solution is very similar to that for the wave equation on a finite string with fixed ends (section 4.3).   The eigenvalues are  and the corresponding eigenfunctions are any non-zero constant multiples of

Example 4.5.1   (continued)

The ODE for  T(t)  becomes

whose general solution is

Therefore

If the initial temperature distribution  f (x) – g(x)  is a simple multiple of  for some integer n, then the solution for v  is just  .

Otherwise, we must attempt a superposition of solutions.

such that .

The Fourier sine series coefficients are

so that the complete solution for  v(x, t)  is

and the complete solution for  u(x, t)  is

Note how this solution can be partitioned into a transient part  v(x, t) (which decays to zero as t increases) and a steady-state part  g(x)  which is the limiting value that the temperature distribution approaches.

Example 4.5.1   (continued)

As a specific example, let  k = 9, T1 = 100,  T2 = 200,  L = 2  and

f (x) = 145x2 – 240x + 100 , (for which f (0) = 100,  f (2) = 200 and  f (x) > 0  x).

Then

The Fourier sine series coefficients are

The complete solution is

Some snapshots of the temperature distribution (from the tenth partial sum) from the Maple file at "www.engr.mun.ca/~ggeorge/5432/demos/ex451.mws" are shown on the next page.

Example 4.5.1   (continued)

The steady state distribution is nearly attained in much less than a second!

END OF CHAPTER 4

END OF ENGI. 5432!